OPEN ENDED LAB (OEL) REPORT: CLO1 & CLO2

ECG Signal Processing & Heart Rate Variability (HRV) Analysis System

-----------------------------------------------------------------------------------------------------------------------------

Course Title: Biomedical Signal Processing      |   Student Name: Biomedical Student

Lab Task: Open-Ended Lab Evaluation             |   Roll/ID Number: BME-2026-09

Department: Biomedical Engineering               |   Supervisor: Dr. Eleanor Vance

Date: May 31, 2026                              |   Score: ______ / ______

-----------------------------------------------------------------------------------------------------------------------------

1. Laboratory Objectives

1. Implement and validate a digital biomedical signal processing pipeline for high-frequency noise and baseline wander removal from raw electrocardiogram (ECG) data.
2. Establish R-peak detection accuracy using the adaptive Pan-Tompkins derivative-threshold algorithm.
3. Develop an outlier-rejection engine using localized median thresholding to correct ectopic cardiac beats.
4. Compute and interpret linear (time-domain, frequency-domain) and non-linear (Poincaré plot, Sample Entropy) HRV metrics for physiological homeostasis assessment.

2. Physiological Background & Introduction

The autonomic nervous system (ANS) controls the sinus node pacemaker through antagonistic sympathetic (stimulatory) and parasympathetic/vagal (inhibitory) branches. Heart Rate Variability (HRV) is the fluctuation in time intervals between consecutive heartbeats (R-R intervals). Depressed HRV markers serve as diagnostic indicators for autonomic neuropathy, diabetic dysautonomia, stress-induced cardiovascular remodeling, and ischemic risks. Accurate analysis requires high-fidelity digital filtering and R-peak extraction, as small noise remnants or ectopic ventricular contractions (PVCs) drastically distort HRV statistics.

3. Filter Design & DSP Methodology

3.1 Baseline Wander Removal (Dual Median Filter)
Baseline drift (0.1-0.5 Hz) caused by respiration is non-linearly tracked using a dual median filter. First, a 200 ms window (removing high-amplitude QRS complexes and T-waves) passes over the ECG. Next, a 600 ms window filters the output to eliminate P-waves. This isolates the baseline wander, which is subtracted from the raw signal.

3.2 P-QRS-T Noise Suppression (Butterworth Bandpass)
To capture ECG waves, a 3rd-order Butterworth bandpass filter is designed with a passband of 0.5 to 40.0 Hz. This band suppresses low-frequency breathing drifts and high-frequency 50/60 Hz powerline interference. It is applied using double-pass filtering (scipy.signal.filtfilt), enforcing zero phase shift to prevent shifting QRS peaks in time.

3.3 QRS R-Peak Detection Method (NeuroKit2 (Default))
R-peak extraction is executed using the 'NeuroKit2 (Default)' algorithm. The Pan-Tompkins method isolates QRS complexes using a derivative stage, a squaring operation, and a moving window integrator (150ms window size), applying adaptive dual-thresholding with back-search and refractory checks. Alternative NeuroKit2 detectors apply advanced mathematical filtering and adaptive peak profiling to locate complexes.

4. Signal Processing Pipeline Results & Figures

Figure 1: Raw vs Filtered ECG Signal (0.5-45 Hz passband)

Figure 2: Pan-Tompkins R-Peak Detection Overlay

Figure 3: Ectopic Beat Detection & Spline Interpolation Correction

Figure 4: R-R Interval Tachogram

Figure 5: Power Spectral Density (Welch Method)

Figure 6: Poincaré Non-Linear Scatter Plot

5. Extracted HRV Parameters & Clinical Metrics

Ectopic Correction Audit: Detected 7 ectopic events out of 69 total beats (10.29% ectopic burden). Ectopic intervals were reconstructed using cubic spline interpolation.

| HRV Parameter | Value | Clinical Reference Range | Physiological Significance |
| --- | --- | --- | --- |
| Mean RR | 821.8 ms | 600 - 1200 ms | Average length of cardiac cycle |
| Mean HR | 73.0 BPM | 50 - 100 BPM | Average heart rate |
| SDNN | 50.0 ms | 30 - 100 ms | Overall autonomic variance (sympathetic & vagal) |
| RMSSD | 53.8 ms | 15 - 80 ms | Vagal (parasympathetic) cardiac deceleration |
| pNN50 | 25.4 % | 1.0 - 40.0 % | Vagal tone / respiratory sinus arrhythmia |
| LF Power | 771.0 ms² | 300 - 2000 ms² | Baroreflex feedback (sympathetic + vagal) |
| HF Power | 1778.1 ms² | 200 - 1000 ms² | Vagal respiratory modulation |
| LF/HF Ratio | 0.43 | 0.5 - 2.0 | Sympathovagal balance (High = stress / Sympathetic) |
| SD1 | 38.3 ms | 15 - 60 ms | Short-term HRV (RMSSD correlate / Parasympathetic) |
| SD2 | 59.4 ms | 40 - 150 ms | Long-term HRV (SDNN correlate / Total variation) |
| Sample Entropy | 1.365 | > 1.0 | Rhythm complexity & cellular homeostatic health |

6. Result Discussion & Autonomic Interpretation

**Autonomic Vagal Assessment (RMSSD = 53.8 ms, SD1 = 38.3 ms):**
High / Robust vagal (parasympathetic) tone. This indicates good cardiovascular fitness, high vagal control, and healthy autonomic recovery.

**Sympathovagal Balance (LF/HF Ratio = 0.43):**
Parasympathetic dominance (LF/HF = 0.43 < 0.5). Typically associated with deep rest, athletic recovery, or high vagal drive.

**Non-Linear Adaptability (Sample Entropy = 1.365, SD2 = 59.4 ms):**
High complexity and healthy physiological adaptability. The cardiac pacemaker behaves non-linearly, showing normal chaotic fluctuations that allow rapid adaptation to external stimuli.

7. Conclusion

This experiment demonstrated the design and validation of a complete biomedical signal processing system. The dual-median filter successfully tracked non-linear baseline drift. Zero-phase bandpass filtering suppressed powerline noise without shifting R-peak coordinates. Adaptive thresholding in the Pan-Tompkins algorithm maintained high detection accuracy. Ectopic beat correction isolated and repaired premature beats. The computed time, frequency, and non-linear HRV parameters accurately quantify cardiac autonomic control, providing essential diagnostics for sympathovagal balance.

8. References

1. Pan, J. and Tompkins, W. J., 'A Real-Time QRS Detection Algorithm,' IEEE Transactions on Biomedical Engineering, vol. BME-32, no. 3, pp. 230-236, 1985.
2. Task Force of the European Society of Cardiology and the North American Society of Pacing and Electrophysiology, 'Heart rate variability: standards of measurement, physiological interpretation, and clinical use,' Circulation, vol. 93, no. 5, pp. 1043-1065, 1996.